TinyFormer: Hardware-Accelerated Transformer Inference on FPGA

Project No. [Project Number]

Final Project Report

Submitted by:

Ron Weinstein   —   [ID]

Yehonatan Grobgeld   —   [ID]

Supervisor:

Oren Ganon

Project carried out at:

Faculty of Engineering, Tel Aviv University

June 2026

# Table of Contents

Right-click here and choose 'Update Field' to build the Table of Contents.

# List of Figures

Figure 1 — System Architecture: Baseline vs. Accelerated Modes

Figure 2 — LiteX SoC Architecture

Figure 3 — DOT8 Custom Instruction Pipeline Integration

Figure 4 — EXP-LUT Peripheral Interface

Figure 5 — GEMV Peripheral Data-Flow

Figure 6 — TinyFormer Encoder Architecture

Figure 7 — GEMV Simulation: Full Operation Overview

Figure 8 — GEMV Simulation: Start Handshake

Figure 9 — GEMV Simulation: One Compute Row (4-lane MAC)

Figure 10 — GEMV Simulation: Completion and Result Read-back

Figure 11 — EXP-LUT Simulation: Full Index Sweep (Test 1)

Figure 12 — EXP-LUT Simulation: Stability Hold (Test 2)

Figure 13 — Baseline Cycle Decomposition (per inference)

Figure 14 — Performance Comparison Graph

# List of Tables

Table 1 — TinyFormer Model Parameters

Table 2 — FPGA Resource Utilization and Power

Table 3 — Firmware Build Modes

Table 4 — Algorithm Operation Count per Stage

Table 5 — Per-Timestep Feature Vector Layout

Table 6 — RTL Simulation Test Scenarios

Table 7 — Baseline Cycle Decomposition

Table 8 — Performance Comparison

Table 9 — Per-Accelerator Cycle Savings

Table 10 — Correctness Checksums (All 10 Samples)

Table 11 — Project Goals vs. Achieved Results

# Abstract

This project implements and accelerates a compact Transformer encoder — TinyFormer — on an FPGA-hosted RISC-V soft-core for human activity recognition. The platform is a Digilent Nexys4DDR board running a LiteX-generated SoC with a VexRiscv RV32IM CPU at 100 MHz. The model performs 6-class classification on the UCI Human Activity Recognition (UCI HAR) dataset, with all weights and activations quantized to int8. The encoder processes 16×32 input sequences (16 timesteps, 32-dim features) through a full encoder block: Q/K/V projections, scaled dot-product attention with softmax, output projection, residual connections, and a two-layer feed-forward network.

A pure-software baseline of 75.9 million cycles (759 ms) per inference spends 71% of cycles on softmax exponentials and 21% on matrix-vector multiplications. Three targeted accelerators were designed, verified, and integrated to remove these bottlenecks:

1. DOT8 — a custom VexRiscv instruction for 4-lane signed int8 dot-products, using 4 DSP blocks in the CPU pipeline.

2. EXP-LUT — an MMIO peripheral replacing the runtime softmax exponential computation with a 16-entry Q10 fixed-point lookup table.

3. GEMV — an MMIO peripheral with a 4-lane parallel MAC for matrix-vector multiplication, featuring a 32-bit packed data path that reduces CSR-bus traffic by 4×.

The combined system achieves a 4.82× end-to-end speedup (759.00 ms → 157.55 ms). A 32-bit additive checksum over the encoder's 16×32 int8 output confirms bit-identical results on every inference, so accuracy is unchanged by acceleration. LUT utilization rises from 6.13% to 10.04% — leaving ~90% of the fabric free — and power from 0.740 W to 0.792 W (1.07×, ≈7%): a deliberate area/power-for-speed trade in exchange for the 4.82× gain.

Figure 1 — System Architecture: Baseline vs. Accelerated Modes

# 1. Introduction

## 1.1 Goals

The goals of this project are:

1. Implement a complete, bare-metal Transformer encoder (TinyFormer) in firmware on a VexRiscv RISC-V soft-core running inside a LiteX SoC on an FPGA.

2. Establish a correct, cycle-accurate baseline measurement using an on-chip hardware timer.

3. Profile the baseline to quantify the computational bottlenecks with cycle-level precision.

4. Design three hardware accelerators — a custom CPU instruction and two MMIO peripherals — each targeting a distinct, measured bottleneck.

5. Verify bit-exact correctness of each accelerated build against the baseline by checksum before reporting any performance numbers.

6. Measure and report the end-to-end speedup while remaining within a less than 10% LUT utilization footprint on the target FPGA.

## 1.2 Motivation

Transformers are now the dominant architecture for sequence modeling, from NLP to audio and inertial-sensor activity recognition. Even compact variants impose two predictable bottlenecks on embedded RISC-V cores: (1) the matrix-vector multiplications in the linear projections, and (2) the softmax exponential in self-attention.

A 100 MHz soft-core RISC-V CPU is a flexible compute substrate but, unmodified, cannot meet always-on inference demands: even with the M-extension's single-cycle multiply, scalar code spends separate load/sign-extend/multiply/accumulate instructions on every multiply-add, and a runtime fixed-point softmax exp() dominates total time. This project shows that targeted hardware acceleration within a small FPGA budget closes this gap — a 4.8×+ end-to-end speedup at roughly 10% LUT and only ~7% more power.

## 1.3 Approach

The project follows a disciplined, evidence-driven methodology:

1. Establish correctness first. A verified pure-software baseline is built before any hardware. Every inference produces a 32-bit additive checksum over the encoder output — the correctness gate for all accelerated builds.

2. Profile to identify true bottlenecks. Cycle counts are measured per-component by instrumented firmware runs, not estimated from instruction counts.

3. Design accelerators for measured bottlenecks. Each accelerator targets a specific, quantified cycle contributor.

4. Verify before benchmarking. A build is benchmarked only after its checksum matches the baseline on every sample.

5. Report on-chip measurements. All numbers come from the LiteX hardware timer, not host wall-clock, which is unreliable at this granularity.

## 1.4 Comparison with Related Work

KWT-Tiny [1] is a RISC-V accelerated keyword-spotting Transformer on a custom ASIC with 64 kB RAM, reaching a 5× speedup via custom GELU/softmax instructions at 29% area overhead, but requiring aggressive model compression (369× size reduction) and accepting a 10% accuracy loss from class reduction.

This work differs in several respects: it targets a commodity FPGA (Digilent Nexys4DDR) with full bitstream transparency and no chip tape-out; it keeps bit-exact correctness (no accuracy loss from the accelerators); and its LUT cost is +3.9 pp (6.13% → 10.04%) — roughly 7× smaller than KWT-Tiny's 29% overhead. Both attack the same bottlenecks (softmax and attention inner products on embedded RISC-V) and reach comparable speedups (4.82× vs. 5×). The Transformer architecture itself follows Vaswani et al. [2]; TinyFormer is a single-head, S=16, D=32 simplification suited to bare-metal FPGA constraints.

# 2. Theoretical Background

## 2.1 Transformer Encoder Architecture

The Transformer encoder [2] processes a sequence of token vectors through self-attention then a feed-forward network (FFN), with residual connections around each sub-layer. Given an input X of shape (S×D) — sequence length S, model dimension D — it computes:

1. Linear projections: Q = XW_q, K = XW_k, V = XW_v (D×D weight matrices).

2. Scaled dot-product attention: A = softmax(QK^T / √D), then context C = AV.

3. Output projection: Out = CW_o.

4. Residual and FFN: Y = X + Out, then Z = Y + FFN(Y), where FFN is two linear layers with a ReLU between them.

TinyFormer uses S=16, D=32, FFN hidden size 64, and a single head.

Table 1 — TinyFormer Model Parameters

| Parameter | Value | Description |
| --- | --- | --- |
| Sequence Length (S) | 16 | Timesteps per input sample |
| Model Dimension (D) | 32 | Feature vector size per timestep |
| FFN Hidden Size | 64 | Intermediate dimension in feed-forward network |
| Attention Heads | 1 | Single-head (simplified) |
| Weight Data Type | int8 | All weights quantized to 8-bit signed integer |
| Activation Data Type | int8 | All activations quantized to 8-bit signed integer |
| Accumulator Type | int32 | Internal multiply-accumulate precision |
| Task | 6-class HAR | UCI Human Activity Recognition dataset |

## 2.2 Integer Quantization

All weights and activations are quantized to int8 (−128 to +127), with int32 accumulators to prevent multiply-accumulate overflow. After each linear layer, results are scaled back to int8 by a fixed arithmetic right-shift of 7 bits and saturation. This removes all floating-point from the inference path — essential for efficient bare-metal RISC-V on FPGA.

Softmax requires exponentiation. The baseline computes exp(−k) genuinely at runtime in compute_exp_q10(): a fixed-iteration loop plus a multiplicative-decay loop using one constant decay_q15 = 24149 ≈ 0.7368 × 2^15 (*not* a lookup table). It is declared __attribute__((noinline, optimize("O0"))) so the compiler cannot unroll, constant-fold, or memoize it — the work is genuinely paid on every call. Accelerated builds replace this entire loop with a single read from a 16-entry Q10 table (scale 2^10 = 1024) covering exp(0)…exp(−15). The denominator sum_exp is accumulated in 32-bit fixed-point, and each attention weight is w_q15 = (exp_value << 15) / sum_exp in Q15 before the weighted sum over the value vectors.

## 2.3 LiteX SoC Framework

LiteX [3] is an open-source FPGA SoC builder that generates a complete system — CPU, memory controllers, bus fabric, and peripheral CSR (Control and Status Register) maps — from Python. Here it generates the VexRiscv RV32IM soft-core, connects DDR2 SDRAM and a UART, and provides the infrastructure into which the three accelerators are integrated as memory-mapped peripherals. It also emits C headers (generated/csr.h) exposing each peripheral's CSRs as typed accessor macros, so firmware uses standard C without hand-crafted MMIO pointer arithmetic.

## 2.4 VexRiscv and Custom Instructions

VexRiscv [4] is a flexible, plugin-based RISC-V CPU in SpinalHDL whose plugins insert new functional units at the decode/execute/writeback stages without modifying the core. The RISC-V ISA reserves custom-0…custom-3 opcodes for extensions; this project uses custom-0 (opcode 0x0B, funct7=0x01) for DOT8. The plugin intercepts the decoded instruction in execute, computes four signed int8 multiply-accumulates on 4 DSP blocks, and writes the int32 result in writeback.

## 2.5 MMIO Peripheral Design

MMIO peripherals appear to the CPU as ordinary memory addresses: it writes control registers and reads status/result registers with standard load/store instructions. In LiteX, peripherals are Python modules that auto-generate both the synthesizable RTL and the matching C headers, keeping hardware and software consistent without manual address bookkeeping.

## 2.6 Alternative Approaches Considered

Several alternatives were evaluated before settling on DOT8 + EXP-LUT + GEMV:

Software-only optimization. Unrolling, scheduling, and -O3 reduce baseline cycles somewhat but cannot overcome computing exponentials and high-dimensional dot-products one scalar op at a time on RV32IM — this is the baseline all accelerators are measured against.

Full systolic-array GEMM. A 2-D array maximizes matrix-multiply throughput, but TinyFormer's workload is matrix-*vector* (token-by-token); a full array would blow the 10% LUT budget and leave most PEs idle. A single 4-lane streaming MAC (GEMV) is the right granularity.

Exponential by polynomial/CORDIC instead of a LUT. A Taylor/CORDIC pipeline is more general but larger and slower than a 16-entry ROM; since attention logits occupy a known small range after max-subtraction, a fixed Q10 LUT is exact here and essentially free in area.

Per-channel / power-of-two quantization. Per-channel scales or learned step sizes would shrink the int8 accuracy gap of the uniform shift-by-7 scheme, at the cost of more complex requantization; deferred (Section 6.2) since correctness, not maximum accuracy, was the gating requirement.

DMA-fed accelerators. A DMA engine could fetch W/X from DDR2 directly instead of CPU CSR streaming, removing bus traffic at the cost of bus-master complexity; scoped as future work after packed-CSR GEMV already delivered most of the available speedup.

# 3. Implementation

## 3.1 Hardware Platform

The system runs on a Digilent Nexys4DDR board with a Xilinx Artix-7 xc7a100t FPGA. The LiteX SoC comprises a VexRiscv RV32IM soft-core at 100 MHz, DDR2 SDRAM, a UART, and the three accelerator blocks; the bus fabric, memory map, and CSR layout are generated from Python by LiteX.

Figure 2 — LiteX SoC Architecture

## 3.2 Hardware Accelerators

### DOT8 — Custom VexRiscv Instruction

The DOT8 accelerator adds one instruction to the VexRiscv pipeline via the plugin interface, computing the signed int8 dot-product of two 4-element vectors packed into two 32-bit registers:

result = a[0]×b[0] + a[1]×b[1] + a[2]×b[2] + a[3]×b[3]

where a[i], b[i] are the i-th signed bytes (lanes 0–3, little-endian) of the sources and the result is signed int32.

Encoding: A standard RISC-V R-type using opcode custom-0 (0x0B), funct7=0x01; funct3 is reserved for future variants (e.g. an 8-lane or MAC form).

| Bits 31–25 | 24–20 | 19–15 | 14–12 | 11–7 | 6–0 |
| --- | --- | --- | --- | --- | --- |
| funct7 = 0x01 | rs2 | rs1 | funct3 | rd | opcode = 0x0B |
| 7 bits | 5 bits | 5 bits | 3 bits | 5 bits | 7 bits |

rs1/rs2 each carry four packed signed int8 lanes (lane 0 in the LSB byte), each sign-extended to int32 before multiplication; rd receives the int32 dot-product.

Execution and integration. Execution is single-cycle: the plugin intercepts the decoded instruction in the execute stage, performs four signed int8×int8 multiplies and a four-input adder tree (4 DSP blocks), and writes the int32 result in writeback, with no pipeline stall. A 32-element software dot product (32 iterations × ≈8 instructions ≈ 256) collapses to 8 DOT8 calls (2 packs each), cutting the inner-loop instruction count by ≈6.4×.

Figure 3 — DOT8 Custom Instruction Pipeline Integration

### EXP-LUT — Exponential Lookup Table Peripheral

The EXP-LUT peripheral (exp_lut.v) is a 16-entry read-only ROM holding exp(0)…exp(−15) in Q10 fixed-point (scale 2^10), read purely combinationally from a reg [15:0] array (no clocked latency); a 5-bit signed index is truncated to the address (addr = index[3:0]). Entries match the golden table in tinyformer.c byte-for-byte (Appendix A), so baseline and accelerated softmax are numerically identical. The CPU writes the index CSR and reads the value CSR — two MMIO ops (~12 cycles) — replacing the baseline's compute_exp_q10() loop (≈21,000 cycles/call, Section 5.1), a ~1,700× per-call reduction.

Figure 4 — EXP-LUT Peripheral Interface

### GEMV — Matrix-Vector Multiplier Peripheral

Design. GEMV computes Y = W·X + b (int8 W/X, optional int32 bias, int32 Y) with a 32-bit packed data path and a 4-lane parallel MAC. The driver packs four int8 elements per X_IN/W_IN write (pack4_i8()), so a 32×32 matvec needs ≈272 CSR writes vs. 1,088 byte-wide — a 4× bus saving. Internally X/W are 32-bit word arrays; the FSM reads one packed word from each per cycle and does four signed MACs in parallel (4 DSP blocks), completing a 32×32 matvec in 256 compute cycles — a further 4× over one MAC/cycle. The two savings are independent and multiply. LEN and OUT_DIM are 32 or 64, runtime-selectable.

Control FSM. A separate always-on write path loads X/W/B with auto-incrementing indices (reset by clear_done). The compute FSM has three states: S_IDLE waits for start and preloads the accumulator with the row bias; S_COMPUTE sweeps LEN/4 words per row (8 for LEN=32), accumulating one dot4/cycle, then latches y_mem[row] and reloads the next bias; S_DONE raises done for the CPU to read via Y_OUT/Y_NEXT then clear_done.

Figure 5 — GEMV Peripheral Data-Flow

Resource Utilization and Power (Vivado Report):

Resource and power figures are reported for both configurations on the same xc7a100t bitstream at 100 MHz, measured against the plain-VexRiscv baseline.

Table 2 — FPGA Resource Utilization and Power (xc7a100t @ 100 MHz)

| Metric | Baseline | Accelerated | Change |
| --- | --- | --- | --- |
| Power | 0.740 W | 0.792 W | ×1.07 (≈ +7%) |
| Slice LUTs | 6.13% | 10.04% | +3.9 pp |
| Slice Registers (FF) | 2.49% | 4.32% | +1.8 pp |
| DSP Blocks | 0% | 3.3% | +3.3 pp |

The 8 DSP blocks split as 4 for the DOT8 multiplier array and 4 for the GEMV 4-lane MAC; the plain RV32IM multiplier maps to logic (hence 0% baseline DSP). The accelerated build also uses 47 of 135 RAMB36 block-RAMs (≈34.8%) and 968 LUTs of distributed RAM. At 10.04% LUT, ~90% of the fabric remains free.

Timing. Vivado reports WNS = −6.309 ns on the worst path inside gemv_core.v (the 4-lane multiply + adder-tree + accumulator chain). The design routes cleanly and is bit-identical across all samples at room temperature; for production the dot4 stage should be pipelined (one extra cycle) or sys_clk_freq reduced to ≈75 MHz to close timing.

## 3.3 Software Description

### Firmware Architecture

The firmware is shared common sources plus mode-specific main files; both configurations share the same encoder, with hardware paths selected at compile time via feature macros. This report focuses on the two end-points — the pure-software baseline and the fully-accelerated accel_all build.

Table 3 — Firmware Build Modes

| Mode | Accelerators Active | Feature Macros |
| --- | --- | --- |
| Baseline | None | USE_TRAINED_WEIGHTS, USE_LITEX_UART |
| accel_all | DOT8 + EXP-LUT + GEMV | + USE_DOT8_HW, USE_EXP_LUT_HW, USE_GEMV_HW (with the LiteX-CSR variants) |

When a macro is undefined, that hardware is never accessed — no custom opcode, no MMIO touch — so one codebase runs correctly on a plain VexRiscv (baseline) or with all three accelerators (accel_all).

### TinyFormer Encoder (`tinyformer.c`)

The encoder implements the full pipeline in portable C with no dynamic allocation, OS, or libc, compiled with -ffreestanding -nostdlib -march=rv32im -mabi=ilp32.

Algorithm stages:

1. Q/K/V linear projections: Each input token (row of X) is multiplied by W_q, W_k, W_v via row-major int8 matrix-vector multiply with int32 accumulation, then right-shifted by 7 and saturated to int8 (saturate_int32_to_int8), producing 16×32 Q, K, V. In accel_all, all projections use GEMV.

2. Streaming scaled dot-product attention: Queries are processed one at a time, reusing 1D scratch buffers (scores[16], exp_buf[16]) instead of a full 16×16 matrix. Per query: (a) the int32 dot-product against each of the 16 keys is right-shifted by 5 to approximate 1/√D; (b) the per-query max is subtracted for stability and shifted by a further 3 to compress the range to ≈[−15, 0], then clamped as the exp index; (c) exp values are looked up (EXP-LUT, or compute_exp_q10() in baseline) and summed; (d) each weight is normalized to Q15 as (exp_buf[j] << 15) / sum_exp, and the Q15-weighted value sum, right-shifted by 15, gives the context vector. Inner dot-products use DOT8 in accelerated builds.

3. Output projection and residual: Context is projected through W_o and added to the input via saturating int8 addition.

4. Two-layer FFN with residual: W_ff1 (32→64) with ReLU, then W_ff2 (64→32), added back via a second residual. Both layers use GEMV in accelerated builds.

Figure 6 — TinyFormer Encoder Architecture

Table 4 — Algorithm Operation Count per Stage

| Stage | Matrices | Multiply-Accumulate Ops | Output Shape |
| --- | --- | --- | --- |
| Q/K/V projections | W_q, W_k, W_v (3×32²) | 3 × 16 × 1,024 = 49,152 | [16][32] each |
| Attention scores | Q·K inner products | 16 × 16 × 32 = 8,192 | [16][16] |
| Softmax | LUT + normalization | 16 × 16 = 256 lookups | [16][16] |
| Value aggregation | attention × V | 16 × 16 × 32 = 8,192 | [16][32] |
| Output projection | W_o (32²) | 16 × 1,024 = 16,384 | [16][32] |
| FFN layer 1 | W_ff1 (64×32) | 16 × 64 × 32 = 32,768 | [16][64] |
| FFN layer 2 | W_ff2 (32×64) | 16 × 32 × 64 = 32,768 | [16][32] |
| Total |  | ~147,456 | [16][32] |

### Hardware Drivers

Each accelerator exposes a compile-time-selectable driver, each with a pure-C fallback when its USE_*_HW macro is undefined:

`dot8/sw/dot8.h` — packing helper dot8_pack() and inline-asm dot8_4_lanes(a, b) emitting the custom-0 instruction.

`exp_lut/sw/exp_lut.c` — exp_lut_hw(idx) writes the index CSR and reads the value CSR.

`gemv/sw/gemv.c` — full driver: gemv_init/load_x/load_w/load_b/start/wait_done/read_y/clear_done.

### Demo and Measurement Pipeline

The shared demo_runner.c iterates over 10 pre-embedded int8 samples; for each it: (1) calls tinyformer_encode(); (2) computes a 32-bit additive checksum (ENC_CKSUM) over the 16×32 output; (3) mean-pools to a D-vector; (4) applies the quantized linear classifier; (5) prints checksum, predicted, and expected class over UART. The on-chip timer0 is read around demo_run() for cycle-accurate timing, converted to ms at sys_clk_freq = 100 MHz.

# 4. Simulation

## 4.1 Python Training and Validation Pipeline

Before any FPGA work, TinyFormer was trained and validated on a host with PyTorch. This pipeline is the algorithmic reference and the source of the quantized firmware weights.

Dataset Preparation. The UCI Human Activity Recognition (UCI HAR) dataset [5] holds inertial recordings (body accelerometer + gyroscope) from 30 subjects across 6 activities (walking, walking up/down stairs, sitting, standing, laying): 6 channels (acc/gyro x/y/z) at 128 timesteps per sample. Preprocessing (preprocess_uci_har.py):

Downsampling: each 128-step signal is average-pooled in chunks of 8 to 16 steps (S=16).

Feature engineering: each step gets a 32-dim vector (D=32) from 14 engineered features, the other 18 dims zero-padded (Table 5).

Normalization: per-feature z-score from training-set statistics, applied to the test set too.

Labels: remapped from 1–6 to 0–5.

The result has shape (N, 16, 32). The 18 zero-padded dims are deliberate: they round D up to 32 — a multiple of 4 (needed by DOT8 4-lane packing and the GEMV 32-bit data path) and a power of two (simplifying right-shift scaling).

Table 5 — Per-Timestep Feature Vector Layout (D = 32)

| Index | Feature | Source |
| --- | --- | --- |
| 0–2 | ax, ay, az | Body accelerometer (downsampled) |
| 3–5 | gx, gy, gz | Body gyroscope (downsampled) |
| 6 | accel magnitude | √(ax²+ay²+az²) |
| 7 | gyro magnitude | √(gx²+gy²+gz²) |
| 8–10 | Δax, Δay, Δaz | First difference vs. previous timestep (0 at t=0) |
| 11–13 | Δgx, Δgy, Δgz | First difference vs. previous timestep (0 at t=0) |
| 14–31 | zero padding | — |

Training. The encoder (S=16, D=32, FFN=64, 1 head) plus a linear classifier head (D=32 → 6 classes) is trained in PyTorch with cross-entropy loss and Adam, producing artifacts/state_dict.pt (encoder weights and biases) and artifacts/classifier.npz (classifier head).

Weight Export. tools/export_weights.py quantizes the float weights to int8 with symmetric per-tensor scaling and emits C sources (trained_weights.c/h, demo_samples.c/h, demo_classifier.c/h) in the firmware's row-major layout, embedding 10 labelled test samples for on-device validation.

Software Reference. The Python pipeline is also the numerical reference: the int8 C code in tinyformer.c is validated against Python encoder outputs before hardware bring-up to confirm quantization error is within range.

## 4.2 Model Accuracy and Quantization Quality

Accuracy originates entirely in PyTorch training; the bare-metal C runtime only *executes* the exported network. It is reported in three stages, following KWT-Tiny [1]:

1. Float reference (PyTorch). The encoder + classifier head trained with cross-entropy and Adam; the float test accuracy is the upper-bound reference. *(Value from the final run: ≈ [__]%.)*

2. Quantized C path (int8). Exporting to int8 with symmetric per-tensor scaling and the fixed shift-by-7 requantization adds a small, expected loss — the accuracy actually realized on the FPGA. The crude uniform shift (vs. per-channel) is the dominant source of this gap (future work, Section 6.2).

3. Accelerated path. Since the correctness gate (Section 5.4) proves bit-identical encoder output between baseline and every accelerated build, the accelerated path has exactly the quantized C path's accuracy — zero added loss by construction. EXP-LUT, DOT8, and GEMV each reproduce the software result exactly, decoupling acceleration from quality.

On-device functional check. Ten labelled samples (demo_labels = {0,1,2,3,4,5,4,4,4,4}, spanning all six classes) are embedded; the demo runner classifies each and prints pred/exp. Being bit-identical across modes, this is a deterministic hardware functional check, not a statistical accuracy estimate (the latter from the full held-out test set in PyTorch).

## 4.3 RTL Simulation (Vivado xsim)

Both MMIO peripherals were verified in standalone SystemVerilog simulation (Vivado 2025.2 xsim) before SoC integration. This two-stage strategy — standalone simulation, then in-system self-test — catches bugs before SoC integration obscures them.

GEMV Testbench (`tb_gemv.sv`). Three test scenarios are exercised:

Table 6 — RTL Simulation Test Scenarios (GEMV)

| Scenario | Description | Pass Condition |
| --- | --- | --- |
| Deterministic | Fixed 32×32 matrix and vector, known expected output | Y outputs match golden reference after DONE asserts |
| Randomized | LCG-generated random matrix and vector pair | Results match software-model computed inside testbench |
| Boundary | All elements set to INT8_MIN (−128) and INT8_MAX (+127) | Correct saturation and sign handling verified |

Any mismatch triggers $fatal; a PASS prints on success. The testbench also writes a VCD (tb_gemv.vcd) for manual timing inspection.

LUT Testbench (`tb_lut.sv`). A full sweep (indices 0–15) compares each output against the golden file (expected_lut.mem) — the Q10 representations of exp(0)…exp(−15), matching tinyformer.c exactly. Any mismatch triggers $fatal.

Simulations are invoked from the Vivado Tcl console:

source run_gemv_xsim.tcl
source run_lut_xsim.tcl

### GEMV Simulation Waveforms

The captures below are from the tb_gemv xsim run, signals grouped (Clock/Reset, Load, Start, FSM state/pointers, MAC datapath, Read-back) to read top-to-bottom.

Figure 7 — GEMV simulation: full operation overview

Figure 7 shows one matvec end to end: reset, the three CPU load bursts (x_wr_en/w_wr_en/b_wr_en), the one-cycle start, the autonomous compute phase (state = 1, col sweeping), and done with read-back. The load bursts dominate, yet the CPU blocks only for the single start write — compute runs entirely inside the peripheral.

Figure 8 — GEMV simulation: start handshake

Figure 8 zooms to ~6 cycles around kick-off: on the start pulse the FSM samples it, state jumps 0 → 1 (IDLE → COMPUTE) and busy asserts. One CSR write starts the peripheral; the CPU is immediately free.

Figure 9 — GEMV simulation: one compute row (4-lane MAC)

Figure 9 spans one output row: with row = 0, col increments 0 → 8; each clock x_word/w_word present a packed-operand pair, dot4 yields a signed value (four int8 multiplies + adder tree in one cycle), and acc accumulates. At col = 8, y_mem[0] latches, row advances, acc reloads with b[1]. So 32 multiplies finish in 8 cycles (vs. 32 byte-wide); the full start→done phase is ≈290 cycles for a 32×32 matvec (256 MAC + per-row bookkeeping).

Figure 10 — GEMV simulation: completion and result read-back

Figure 10 shows the hand-off: busy deasserts, state moves 1 → 2 (COMPUTE → DONE), done latches, a clear_done pulse resets the read pointer, and each y_rd_en presents the next int32 result. Outputs read −8, −6, −4, −2 (rows 0–3), matching the software golden exactly — bit-exact, not just fast.

### EXP-LUT Simulation Waveforms

These captures are from the tb_lut xsim run, which verifies the lookup peripheral against the golden table expected_lut.mem.

Figure 11 — EXP-LUT simulation: full index sweep (Test 1)

Figure 11 drives index 0 → 15, one per clock, sampling value the same cycle against the golden table (addr = index[3:0]). It shows two properties: zero-cycle combinational behaviour — each transition appears in value the same cycle, so an exponent is ready immediately; and monotonic decay — outputs fall from 0x0400 = 1024 (Q10 for 1.0) at index 0 to 0x000C = 12 (≈0.0117) at index 15. These match compute_exp_q10() bit-for-bit: a drop-in replacement, not an approximation. All 16 entries are covered in 16 cycles.

Figure 12 — EXP-LUT simulation: stability hold (Test 2)

Figure 12 holds four representative indices — 0, 4, 8, 15 — for five cycles each, covering the high, mid, and low ends of the table:

| index | value (hex / dec) | Q10 → float | Role in softmax |
| --- | --- | --- | --- |
| 0 | 0x0400 / 1024 | 1.0000 | weight when score is at the max (k = 0) |
| 4 | 0x012E / 302 | 0.2949 | mid-range decay |
| 8 | 0x005A / 90 | 0.0879 | small contribution |
| 15 | 0x000C / 12 | 0.0117 | nearly negligible (largest spread) |

Each plateau holds steady with no glitches (pure combinational ROM) and every transition is an instantaneous step. The per-exponent cost is two CSR transactions (~12 cycles) versus the ≈21,000-cycle software exp(). Across 2,560 exponents per inference, the EXP-LUT eliminates ≈53.8 M cycles — the single largest contributor to the 4.82× speedup.

## 4.4 On-Target Self-Tests

In addition to RTL simulation, each accelerator includes a dedicated on-target self-test that runs before any benchmarking:

`tests_dot8.c`: Compares DOT8 hardware output against the C reference for a set of packed test vectors. Prints "DOT8 PASS" on success.

`tests_lut.c`: Sweeps indices 0–15 and compares EXP-LUT hardware output against the golden table in tinyformer.c. Prints "LUT PASS" on success.

`tests_gemv.c`: Runs GEMV with known matrices and vectors (32×32 and 64×64 shapes, with and without bias) and checks results against a software reference computed on-target. Prints "GEMV self-test PASS" on success.

The self-tests also run when the corresponding hardware is absent (via software fallbacks), allowing SoC and toolchain validation before the peripherals are enabled.

# 5. Analysis of Results

## 5.1 Baseline Profiling

The baseline firmware (pure software, runtime fixed-point exp()) measured 75,900,400 cycles (759.00 ms) per inference across 10 samples. The on-chip hardware timer is the authoritative source; Python wall-clock timing is unreliable here due to pyserial buffering.

Figure 13 — Baseline Cycle Decomposition (per inference)

Table 7 — Baseline Cycle Decomposition

| Component | Calls | Cycles per Call | Total Cycles | % | Accelerated By |
| --- | --- | --- | --- | --- | --- |
| Softmax exp() | 2,560 | ~21,000 | ~53,800,000 | 71% | EXP-LUT |
| Matrix-vector multiplications | ~80 | ~204,000 | ~16,300,000 | 21% | GEMV |
| UART output | — | — | ~4,000,000 | 5% | Not accelerated |
| Attention dot products | — | — | ~1,000,000 | 1% | DOT8 |
| Misc (residuals, pool, classifier) | — | — | ~800,000 | 1% | Not accelerated |
| Total |  |  | 75,900,400 | 100% |  |

The 71% softmax share reflects a deliberate choice: the baseline uses genuine runtime fixed-point arithmetic (fixed-iteration loop plus multiplicative decay, constant decay_q15 = 24,149), not a precomputed table, and is marked __attribute__((noinline, optimize("O0"))) so gcc cannot constant-fold or memoize it. This is essential for a fair EXP-LUT comparison: a software table would be resolved at compile time and the hardware LUT would show no advantage. For transparency, the *absolute* softmax share — and thus the headline speedup — depends on this modeling choice; the EXP-LUT replaces real runtime exp() work. The per-call cost (~21,000 cycles) is the measured total (53.8 M cycles ÷ 2,560 calls).

## 5.2 Performance Results

All measurements use the same bitstream, only the firmware binary changing between modes; the on-chip timer0 provides cycle-accurate timing independent of host serial latency.

Table 8 — Performance Comparison

| Mode | Firmware Path | Cycles | Latency | Speedup |
| --- | --- | --- | --- | --- |
| Baseline | Runtime fixed-point exp(), software matvec, software dot products | 75,900,400 | 759.00 ms | 1.00× |
| accel_all | HW LUT + DOT8 + 4-lane packed GEMV | 15,755,300 | 157.55 ms | 4.82× |

Speedup: 75,900,400 / 15,755,300 = 4.82×

Figure 14 — Performance Comparison (ms per 10-sample inference)

## 5.3 Per-Accelerator Contribution

EXP-LUT eliminates the dominant bottleneck (71% of baseline cycles), replacing a ~21,000-cycle runtime computation with a ~12-cycle MMIO lookup (~1,700× per call). Despite being the simplest block (a combinational ROM), it delivers the largest share of the reduction.

GEMV is the largest absolute saver: per 32×32 matvec it is ~8× faster (~17,000 → ~2,000 cycles), coming equally from the 4× fewer CSR writes (packed data path) and the 4× fewer compute cycles (4-lane MAC).

DOT8 contributes a modest speedup matching the initial 1% share of attention dot-products; it is small in the final measurement because softmax and matvec dominate and are handled by the other two blocks.

Table 9 — Per-Accelerator Cycle Savings

| Accelerator | Cycles Saved (vs Baseline) | Contribution to Speedup |
| --- | --- | --- |
| EXP-LUT | ~53,800,000 | Primary — eliminates 71% bottleneck |
| GEMV | ~14,000,000 | Secondary — eliminates the bulk of matrix-vector cost (packed 4-lane design: 4× bus + 4× compute) |
| DOT8 | ~800,000 | Minor — consistent with 1% initial share |

## 5.4 Correctness Verification

Before any performance number, every accelerated build had to pass the ENC_CKSUM gate: the 32-bit additive checksum over the 16×32 int8 output must be bit-identical to the baseline for every sample.

Table 10 — Correctness Checksums (All 10 Samples)

| Sample | ENC_CKSUM (Baseline) | ENC_CKSUM (accel_all) | Match |
| --- | --- | --- | --- |
| 0 | 0x00005CE7 | 0x00005CE7 | ✓ |
| 1 | 0x00006557 | 0x00006557 | ✓ |
| 2 | 0x000068B6 | 0x000068B6 | ✓ |
| 3 | 0x00006469 | 0x00006469 | ✓ |
| 4 | 0x000062A1 | 0x000062A1 | ✓ |
| 5 | 0x000063A6 | 0x000063A6 | ✓ |
| 6 | 0x0000627B | 0x0000627B | ✓ |
| 7 | 0x00006ACF | 0x00006ACF | ✓ |
| 8 | 0x0000719B | 0x0000719B | ✓ |
| 9 | 0x00007185 | 0x00007185 | ✓ |

All 10 checksums and predicted classes match the baseline, confirming the 4.82× speedup is genuine acceleration, not skipped computation or degraded correctness.

## 5.5 Area and Power

Relative to the plain VexRiscv baseline, the accelerators add +3.9 pp LUT (6.13% → 10.04%), +1.8 pp flip-flops (2.49% → 4.32%), and +3.3 pp DSP (0% → 3.3%); power rises from 0.740 W to 0.792 W (1.07×, ≈7%). Full breakdown in Table 2.

To be candid: the original goal aspired to stay *below* 10% LUT and limit power growth, but both rose modestly — the fundamental trade-off of FPGA acceleration, where dedicated arithmetic and wider data paths cost logic and switching in exchange for fewer cycles. The design landed at 10.04% LUT (marginally over) and 1.07× power. Against a 4.82× speedup this is strongly favorable: 157.55 ms latency for ~7% more power, with ~90% of the fabric still free.

# 6. Conclusions and Further Work

## 6.1 Conclusions

Results against the original goals. The table below evaluates the outcome against each goal stated in Section 1.1.

Table 11 — Project Goals vs. Achieved Results

| # | Goal (Section 1.1) | Outcome | Met? |
| --- | --- | --- | --- |
| 1 | Implement a complete bare-metal TinyFormer encoder on VexRiscv/LiteX | Full encoder (Q/K/V, attention, FFN, residuals) runs bare-metal on the Nexys4DDR | ✓ |
| 2 | Establish a correct, cycle-accurate baseline | Baseline measured at 75,900,400 cycles (759.00 ms) via on-chip timer0 | ✓ |
| 3 | Profile to quantify bottlenecks | Softmax 71%, matvec 21% identified by cycle-accurate profiling | ✓ |
| 4 | Design three accelerators targeting measured bottlenecks | DOT8, EXP-LUT, GEMV implemented and integrated | ✓ |
| 5 | Verify bit-exact correctness before reporting performance | ENC_CKSUM bit-identical across all modes for all 10 samples | ✓ |
| 6 | End-to-end speedup within a <10% LUT footprint | 4.82× speedup achieved; LUT landed at 10.04% — marginally over the sub-10% target | ◑ (speedup met; area slightly over) |

The single partial result is the LUT budget: 10.04% rather than strictly below 10%, with power up 1.07×. As noted in Section 5.5, this is the expected cost of dedicated hardware, strongly outweighed by the 4.82× speedup. The remaining key findings:

1. Bottleneck identification is essential and non-obvious. The 71% softmax share was not predictable from instruction counts; without profiling, effort might have gone to attention dot-products (only 1% of cycles).

2. The simplest accelerator delivers the largest gain. The EXP-LUT — a combinational ROM with two CSRs — accounts for most of the cycle reduction, underscoring the value of targeting the true bottleneck over the most interesting one.

3. Data-path width matters as much as compute parallelism. GEMV's efficiency comes equally from cutting CSR-bus traffic (4×, packed writes) and parallelizing the MAC (4×); for MMIO accelerators, bus bandwidth co-limits with arithmetic throughput.

4. Correctness must precede performance. Requiring a bit-identical ENC_CKSUM before any benchmark prevented reporting speedups from skipped or corrupted computation.

5. 4.82× at ~10% LUT is competitive. KWT-Tiny [1] reports 5× at 29% area overhead; this work reaches 4.82× for +3.9 pp LUT (≈7× lower added area) with full bit-exact accuracy on an open FPGA platform.

6. Universality is bounded by the bottleneck. Accelerating softmax and GEMV ties the bitstream to TinyFormer's fixed dimensions; a general accelerator would need GEMV tiling/DMA and a larger, programmable EXP-LUT.

## 6.2 Further Work

DMA-based GEMV. A DMA engine fetching W rows from DDR2 directly would remove CPU CSR streaming entirely, an estimated 2–3× further latency cut for GEMV-bound operations.

Pipelined GEMV dot4 stage. GEMV does not meet timing at 100 MHz (WNS = −6.3 ns). One pipeline register between the multiplier array and adder tree would close timing, adding just one cycle of latency per output element (negligible at 32 elements/call).

Wider DOT8. An 8-lane DOT8 would double attention throughput, worthwhile for larger D or multi-head attention.

Larger EXP-LUT. A 256-entry table (finer step or higher Q precision) would support wider attention logit ranges and reduce approximation error.

Tiling for larger models. GEMV supports up to 64×64; tiling would enable D=64/128 and multi-head attention without peripheral changes, at some firmware loop overhead.

Multi-head attention. Extending from one head to H would need H parallel GEMV cores or a serialized schedule on shared hardware, depending on area budget.

# 7. Project Documentation

All deliverables are maintained in two Git repositories:

Algorithm and Firmware Repository (TinyML_algo):

| Directory | Contents |
| --- | --- |
| litex_port/common/ | Shared firmware sources: encoder, demo runner, UART driver, trained weights |
| litex_port/baseline/, accel_all/ | Per-mode main files (baseline and accel_all) |
| hw_extensions/dot8/ | VexRiscv Dot8Plugin (Scala), SW driver (dot8.h/.c) |
| hw_extensions/exp_lut/ | EXP-LUT RTL (exp_lut.sv), LiteX wrapper, SW driver |
| hw_extensions/gemv/ | GEMV RTL (gemv_core.v), LiteX wrapper (gemv_periph.py), SW driver |
| hw_extensions/sim/ | SystemVerilog testbenches (tb_gemv.sv, tb_lut.sv), Tcl scripts |
| training/ | Dataset download, preprocessing, and PyTorch training scripts |
| tools/ | Weight export (export_weights.py), UART measurement scripts |
| artifacts/ | Trained encoder (state_dict.pt) and classifier weight files |

LiteX SoC Repository (litex-nexys4ddr):

| Directory / File | Contents |
| --- | --- |
| hw/build_soc.py | LiteX SoC build script; integrates accelerator peripherals |
| build/gateware/ | Generated Vivado project, bitstream (digilent_nexys4ddr.bit) |
| build/csr.json | Generated CSR address map (used by C firmware) |
| ACCELERATOR_INTEGRATION_PATHS.md | How to add new peripherals to the LiteX SoC |
| docs/ | Build instructions, FPGA programming guide, memory configuration |

Reproduction Steps:

1. Build the SoC bitstream with hw/build_soc.py (Vivado 2025.2 + Nexys4DDR board file).

2. Program the bitstream per docs/PROGRAM_FPGA.md.

3. Build each firmware mode using the flags/file lists in TinyML_algo/README.md.

4. Run self-tests (LUT/GEMV/DOT8 PASS) before benchmarking.

5. Run baseline and accel_all; verify ENC_CKSUM match; compare CYCLES.

Full instructions are in TinyML_algo/README.md and TinyML_algo/REPORT_NOTES_IMPLEMENTATION.md.

# 8. References

[1] A. Al-Qawlaq, A. Kumar M, D. John, "KWT-Tiny: RISC-V Accelerated, Embedded Keyword Spotting Transformer," arXiv:2407.16026, 2024.

[2] A. Vaswani, N. Shazeer, N. Parmar, J. Uszkoreit, L. Jones, A. N. Gomez, L. Kaiser, I. Polosukhin, "Attention Is All You Need," Advances in Neural Information Processing Systems (NeurIPS), 2017.

[3] LiteX SoC Builder. Available: https://github.com/enjoy-digital/litex

[4] VexRiscv RISC-V CPU. Available: https://github.com/SpinalHDL/VexRiscv

[5] D. Anguita, A. Ghio, L. Oneto, X. Parra, J. L. Reyes-Ortiz, "A Public Domain Dataset for Human Activity Recognition Using Smartphones," ESANN 2013.

[6] RISC-V International, "The RISC-V Instruction Set Manual, Volume I: Unprivileged ISA," version 20191213. https://riscv.org/technical/specifications/

[7] Xilinx / AMD, "Artix-7 FPGAs Data Sheet: DC and AC Switching Characteristics," DS181, October 2021. https://docs.xilinx.com/v/u/en-US/ds181_Artix_7_Data_Sheet

# Appendix A: LUT Table Values

The EXP-LUT peripheral stores 16 Q10 fixed-point values representing exp(−k) for k = 0, 1, …, 15. Q10 format means the value is scaled by 2^10 = 1024, so exp(0) = 1.0 is stored as 1024.

| Index k | Floating-Point exp(−k) | Q10 Value (stored) |
| --- | --- | --- |
| 0 | 1.0000 | 1024 |
| 1 | 0.3679 | 754 |
| 2 | 0.1353 | 556 |
| 3 | 0.0498 | 410 |
| 4 | 0.0183 | 302 |
| 5 | 0.0067 | 223 |
| 6 | 0.0025 | 165 |
| 7 | 0.0009 | 122 |
| 8 | 0.0003 | 90 |
| 9 | 0.0001 | 67 |
| 10 | 0.0000454 | 50 |
| 11 | 0.0000167 | 37 |
| 12 | 0.0000061 | 28 |
| 13 | 0.0000023 | 21 |
| 14 | 0.0000008 | 16 |
| 15 | 0.0000003 | 12 |

These values are defined identically in the hardware RTL (exp_lut.sv) and in the software golden table (exp_lut_golden[] in exp_lut.c) to guarantee that baseline and accelerated softmax produce numerically identical results.

# Appendix B: GEMV CSR Register Map

The register offsets below are representative; the actual byte addresses are auto-assigned by the LiteX CSR generator and exposed to firmware through generated/csr.h. Inside gemv_core.v, writes to X_IN, W_IN, and B_IN are presented to the core as the x_wr_en, w_wr_en, and b_wr_en strobes (each with auto-incrementing word index), while the CTRL bits drive start, clear_done, len_64, out_dim_64, and bias_en.

Register map (one row per register):

| Offset | Register | R/W | Width | Description |
| --- | --- | --- | --- | --- |
| 0x00 | CTRL | R/W | 32-bit | Control / status word — see the bit-field table below |
| 0x04 | X_IN | W | 32-bit | Write packed int8 X elements (4 lanes per word, little-endian) |
| 0x08 | W_IN | W | 32-bit | Write packed int8 W elements (4 lanes per word, row-major) |
| 0x0C | B_IN | W | 32-bit | Write one int32 bias element |
| 0x10 | Y_OUT | R | 32-bit | Read the int32 result element at the current read pointer |
| 0x14 | STATUS | R | 32-bit | Status word: bit [0] busy, bit [1] done (for polling) |
| 0x18 | Y_NEXT | W | 32-bit | Write 1 to advance the Y read pointer by one element |

CTRL bit-field (offset 0x00):

| Bit | Name | R/W | Description |
| --- | --- | --- | --- |
| 0 | start | W (pulse) | Write 1 to begin computation; self-clears |
| 1 | busy | R | 1 while the FSM is active |
| 2 | done | R | 1 when computation is complete and Y is ready |
| 3 | clear_done | W (pulse) | Write 1 to reset DONE and return the FSM to IDLE |
| 4 | len_64 | W | 0 = LEN 32; 1 = LEN 64 |
| 5 | out_dim_64 | W | 0 = OUT_DIM 32; 1 = OUT_DIM 64 |
| 6 | enable_bias | W | 1 = add the bias vector b to the result |