Suggested screenshots for the report

Screenshot 1 — Overview of the whole inference (1 slide, big picture)

- Zoom: 0 ns to ~6200 ns (use Zoom Fit)

- What it shows: Reset → Load (X/W/B blocks visible as long en pulses) → Start → Compute → Done → Read.

- Caption: Full GEMV operation: load → compute → result. Three load bursts dominate the picture but the CPU only

"blocks" during the start pulse — the compute phase runs autonomously.

Screenshot 2 — Start handshake (proof the CPU "kicks off" the peripheral)

- Zoom: 3060 ns – 3120 ns (~6 cycles wide)

- Signals to highlight: start, busy, done, state

- What to see:

- At 3080 ns: start ↑ 1 for exactly one cycle.

- At 3085 ns (posedge during the start pulse): the FSM samples it → state jumps 0→1 (IDLE→COMPUTE) and busy ↑ 1.

- At 3090 ns: start ↓ 0.

- Caption: Single-cycle handshake — one CSR write puts the peripheral in COMPUTE, and the CPU is immediately free.

Screenshot 3 — First compute row (the money shot — proves the 4-lane MAC)

- Zoom: 3085 ns – 3185 ns (~10 cycles, one full row's worth)

- Signals to highlight: state, row, col, x_word, w_word, dot4, acc

- What to see:

- state = 1 (COMPUTE) throughout this span.

- row = 0 throughout this span.

- col increments 0 → 1 → 2 → … → 7 → 8 over 9 clocks.

- x_word and w_word change every clock — these are the two packed operands feeding the MAC.

- dot4 shows a new signed value every clock — that's 4 signed int8 multiplies + adder tree in 1 cycle.

- acc ramps up monotonically as the 4-lane MAC accumulates.

- When col hits 8 (at ~3175 ns): y_mem[0] gets latched, row jumps to 1, col resets to 0, acc reloads with b[1].

- Caption: Inside one row of the matvec: dot4 performs 4 signed int8 multiplies + sum every clock; acc accumulates them.

8 such MACs cover the 32-element row → 32 multiplies in 8 cycles, vs 32 cycles in v1.

Screenshot 4 — Done + read-back (proof of correct output)

- Zoom: 5960 ns – 6080 ns

- Signals to highlight: state, done, busy, clear_done, y_rd_en, y_rd_data

- What to see:

- ~5965 ns: busy ↓ 0, state jumps 1→2 (COMPUTE→DONE).

- ~5975 ns: done ↑ 1 (latched).

- ~5985 ns: clear_done pulse — done goes low, read pointer resets to 0.

- ~6000 ns onwards: y_rd_en pulses 4 times; on each pulse y_rd_data shows the next row's int32 result. In signed

decimal you'll see: -8, -6, -4, -2 (rows 0-3, hand-computable: 1·(-8)+0, 2·(-8)+10, 3·(-8)+20, 4·(-8)+30).

- Caption: Peripheral signals completion to CPU; CPU reads 4 int32 results bit-exactly matching the software golden.

---

How to zoom precisely in Vivado

1. Mark cursors — drop a primary cursor at the start time, and a secondary at the end. Right-click in the wave area →

Zoom Between Cursors.

2. Or by time — in the time field at the top of the wave window, type 3085 ns then 3185 ns to jump.

3. Or by selection — click-and-drag with the middle mouse button across the time region you want to zoom into.

The total cycle count between start ↑ and done ↑ is what you quote in the report as the compute-phase length (~290

cycles, matching what the TB's console printed).