Figure: EXP-LUT — full sweep (Test 1)

▎ The testbench drives index through every

▎ legal value 0 → 15, one per clock cycle,

▎ while the LUT output value is sampled on

▎ the same cycle. Each pair (index, value)

▎ is verified against expected_lut.mem

▎ (golden table).

▎

▎ Key observations:

▎

▎ - Combinational behaviour, zero-cycle

▎ latency. Every index transition is

▎ immediately reflected in value — there is

▎ no pipeline register between input and

▎ output. A softmax exponent is therefore

▎ available on the same cycle the CPU writes

▎  the index.

▎ - Monotonic decay. The 16 outputs descend

▎ smoothly from 0x0400 = 1024 (Q10

▎ representation of 1.0) at index=0, down to

▎  0x000C = 12 (≈ 0.0117) at index=15. This

▎ matches the software softmax helper

▎ compute_exp_q10() in tinyformer.c

▎ bit-for-bit — the EXP-LUT is a drop-in

▎ numerical replacement, not an

▎ approximation.

▎ - Full functional coverage in one

▎ screenshot. All 16 LUT entries are

▎ exercised in 16 cycles. The console log

▎ reports ok swp idx=k for every k,

▎ confirming bit-exact correctness across

▎ the entire address space.

▎

▎ Significance for the project: this single

▎ lookup replaces a ~21,000-cycle software

▎ exp() call in the baseline firmware.

▎ Multiplied by 2,560 softmax exponents per

▎ inference, the EXP-LUT is responsible for

▎ ~53.8 M of the 60 M cycles saved by

▎ accel_all — the single largest

▎ contribution to the 4.82× end-to-end

▎ speedup.

---

Figure: EXP-LUT — stability hold (Test 2)

[tb_lut_v2] --- test 2: hold each of a few

indices for 5 cycles ---

...

[tb_lut_v2] ALL TESTS PASS

If you hit an error (radix name, divider

syntax — same Vivado quirks as before),

paste it and I'll patch.

✻ Cooked for 2m 6s

❯ give me a description for the LUT for each

screenshot. i made two screenshots, for

test 1 and tests 2 all combined

● Here are two captions, written to drop straight into the report under each figure.

---

Figure: EXP-LUT — full sweep (Test 1)

▎ The testbench drives index through every legal value 0 → 15, one per clock cycle, while the LUT output value is sampled on the same cycle. Each pair (index, value) is verified against expected_lut.mem (golden table).

▎

▎ Key observations:

▎

▎ - Combinational behaviour, zero-cycle latency. Every index transition is immediately reflected in value — there is no pipeline register between input and output. A softmax exponent is therefore available on the same cycle the CPU writes the

▎ index.

▎ - Monotonic decay. The 16 outputs descend smoothly from 0x0400 = 1024 (Q10 representation of 1.0) at index=0, down to 0x000C = 12 (≈ 0.0117) at index=15. This matches the software softmax helper compute_exp_q10() in tinyformer.c bit-for-bit — the

▎  EXP-LUT is a drop-in numerical replacement, not an approximation.

▎ - Full functional coverage in one screenshot. All 16 LUT entries are exercised in 16 cycles. The console log reports ok swp idx=k for every k, confirming bit-exact correctness across the entire address space.

▎

▎ Significance for the project: this single lookup replaces a ~21,000-cycle software exp() call in the baseline firmware. Multiplied by 2,560 softmax exponents per inference, the EXP-LUT is responsible for ~53.8 M of the 60 M cycles saved by

▎ accel_all — the single largest contribution to the 4.82× end-to-end speedup.

---

Figure: EXP-LUT — stability hold (Test 2)

▎ Four representative indices are driven and held for five clock cycles each — index = 0, 4, 8, 15 — covering the high, mid, and low ends of the LUT. The output value is captured during the stable plateau of each hold.

▎

▎ | index | value (hex / dec) | Q10 → float | Meaning in softmax                      |

▎ |-------|-------------------|-------------|-----------------------------------------|

▎ | 0     | 0x0400 / 1024     | 1.0000      | weight when score is at the max (k = 0) |

▎ | 4     | 0x012E / 302      | 0.2949      | mid-range decay                         |

▎ | 8     | 0x005A / 90       | 0.0879      | small contribution                      |

▎ | 15    | 0x000C / 12       | 0.0117      | nearly negligible (largest spread)      |

▎

▎ Key observations:

▎

▎ - Stable, deterministic output. Once index is set, value holds rock-steady for the full five cycles with no glitches — the LUT is a pure combinational ROM with no internal state to settle.

▎ - Hand-verifiable values. Each plateau shows a value that can be checked directly against the table in exp_lut.v and the golden file expected_lut.mem. The console prints both hex and decimal forms.

▎ - Zero-cycle response. On every index transition, value jumps to the new entry on the same clock — visible as instantaneous step changes between holds, with no transient or intermediate state.

▎

▎ Significance: confirms that the accelerator delivers a usable softmax weight in 0 cycles of latency, with no setup or pipeline penalty. The CPU's only cost per exponent is the two Wishbone CSR transactions (write index, read value) — roughly 12

▎ cycles total, versus the ~21,000-cycle software baseline.

---